Highlights

- The Dammam Eocene aquifer of the Shigaya D-Field of Kuwait was studied using geophysical and hydrochemical data.

- The aquifer has a heterogeneous is composed of limestone, dolomitized at the top, with dispersed argillaceous materials ranges.

- The total matrix porosity ranges from 4 to 35 %, with the highest values recorded at the top of the formation, that was enhanced by dolomitization, fracturing, and dissolution.

- The groundwater type is Na2SO4, with Na+ and Ca++ and strong acids (SO4-- and Cl-) than the weak acids (CO3-- and HCO3-).

- A simple dissolution or mixing nature took place between sodium ions of the meteoric water and the magnesium and calcium ions of the groundwater.

- A remarkable decrease of the Dammam water salinity was detected after 1977, as a result of groundwater exchanges that took place between the overlying, relatively lower salinity, Kuwait Group aquifer through the wellbores whenever the wells are not being pumped.